EDIH HACKDAY PROPOSAL

Tania B. Lopez Garcia (taniabeatriz.lopezgarcia@hslu.ch) – HSLU
Yousra Sidqi (yousra.sidqi@hslu.ch) - HSLU

Hack day Proposals

Designing pocket and grid-friendly electricity communities

Justification:

Not every vZEV composition performs equally well.

A community with:

- too many PV owners,

- too little flexibility,

- no storage,

- may become “grid-unfriendly” -> simultaneous PV exports, sharp residual load changes, i.e. synthetic peaks. Question: when do vZEVs worsen feeder stress?

Data provided:

- Household smart meter profiles & presence of HP, EV or PV -> aprox. 150 profiles, hourly time steps.

- PV generation profiles.

- Tariff scheme

Considerations:

- Limits for maximum import/export at a given timestep (Pmax).

- Peak load & peak export:

  - Values of maximum peak load and maximum peak export

  - How many time steps does P > 0.9 Pmax

- Ramp rate: Ramp = abs(P(t) – P(t-1))

- Scoring function:

Score = Total LEC savings + α (community self-consumption) – β (peaks) – γ (duration of peaks) –  δ(ramp rate) – λ(limit violations)

Hard penalty: energy balance must be maintained, so for each member:

PV + LEC import + grid import + battery discharge  = load + LEC export + grid export + battery charge

- If this is not respected, solution is not valid.

This way participants try to maximize:

- Savings per household

- Self-consumption

- Local trading

While minimizing:

- Peak load & peak exports

- Ramp rates

- Power constraints violations

Participants can:

- Determine electricity allocation in LEC: a basic way to do this is to assign a constant “ratio” to each member that is proportional to their consumption with respect to other members of the LEC, but other alternatives can be explored.

- Find economically beneficial and grid friendly community design:

  - Community size/load profile compatibility (2-15 buildings possible)

  - PV penetration (%) <- who has PV

  - ESS penetration (%) <- who has ESS

- Design EMS:

  - Load shifting (is it better to have a few highly flexible members? Or many mildly flexible?)

    - According to profile metadata, a percentage of the load will be shiftable (10-40%).

    - If HP or EV are available: time windows will be available for load shifting.

  - Battery optimization: households with batteries can control when to charge/discharge (prioritize self-consumption? local trading? peak shaving?) <- battery size/characteristics/constraints will be provided

Outputs

- methodology for identifying beneficial vZEV design (beneficial for members of vZEV and for the grid/DSO)

- methodology for load shifting/battery operation when part of a vZEV

This could become: a screening tool for evaluating future vZEV requests.